PLANNING DIVISION

STAFF REPORT

Planning and Zoning Meeting                              	January 22, 2025

CASE No.: ZON24-00638                                               		              PROJECT NAME: Lofts at 121

Owner’s Name:	Lofts at 121 LLC

Applicant's Name:	Tim Boyle, Atmosphere Architects

Location of Request:	121 and 129 North Beverly, approximately 881 feet north of the northeast corner of East Main Street and North Beverly.

Parcel No(s):                               135-53-014, 135-53-024A

Request:		Rezone from Single Residence-6 (RS-6) to Multiple Residence-4 with a Planned Area Development Overlay (RM-4-PAD) and Site Plan Review for a multiple residence development.

Existing Zoning District:	RS-6

Council District:                       	4

Site Size:		3± acres

Proposed Use(s):	Multiple Residence

Existing Use(s):	Single Residence

P&Z Hearing Date(s):	January 22, 2025 / 4:00 p.m.

Staff Planner:	Josh Grandlienard, AICP, Senior Planner

Staff Recommendation:	APPROVAL with conditions

HISTORY

On December 21, 1948, the City Council approved the annexation of 2,419± acres of land, including the project site, and subsequently zoned the property to RS-6 (Annexation 60, Ord. No. 228).

PROJECT DESCRIPTION

The applicant is requesting to rezone the project site from Single Residence-6 to Multiple Residence-4 with a Planned Area Development Overlay (RM-4-PAD) and approval of an Initial Site Plan review to allow for a multiple residence development (Proposed Project).

The Proposed Project is located north of Main Street on the east side of North Beverly, on an approximate 3± acre property that is currently developed as a single residence.

The applicant is requesting a PAD overlay to modify development standards to facilitate the development of the proposed 68-unit multiple residence development.

General Plan Character Area Designation and Goals:

The Mesa 2040 General Plan Character Area designation on the property is Neighborhood with a Traditional Sub-type and a Transit Station Area overlay. Per Chapter 7 of the General Plan, the Neighborhood-Traditional Character Type is predominantly single residence in character but may contain a variety of lot sizes and dwelling types. Traditional neighborhoods may also contain supportive land uses such as small, neighborhood-scale offices, retail, restaurants, and personal services that are located along arterials or at the intersection of arterials and collector streets.  Per Chapter 7 of General Plan, the Station Area character type is intended to have more intense development than the associated corridor and are expected to transition into an urban building form.

The RM-4 zoning district is identified as a secondary zoning district and multiple residence is identified as a primary land use for the Traditional Neighborhood Character Area. The RM-4 zoning district is identified as a primary zoning district and multiple residence uses are identified as a primary land use in the Transit Station Area Character Area.

The Proposed Project is consistent with the existing and anticipated development pattern of the area. Properties to the west, east, and south of the subject property are zoned RM-4 and RM-3. Properties to the north of the project site are existing single residence developments. The proposed residential development is consistent with development patterns in the area.

Zoning District Designations:

The applicant has requested to rezone the project site from RS-6 to RM-4-PAD. Per Section 11-5 of the MZO, the RM-4 zoning district allows for a wide range of housing types including multiple residence use with a maximum density of 30 dwelling units per net acre. The Proposed Project has a density of 22.6± dwelling units per acre, which meets the maximum per the MZO.

Surrounding Zoning Designations and Existing Use Activity:

| Northwest (Across North Beverly) RS-6 Single Residences | North RS-6 Single Residences | Northeast RM-4 Multiple Residence |
| --- | --- | --- |
| West (Across North Beverly) RM-3-PAD Multiple Residence | Project Site RS-6 Single Residence | East RM-4 Multiple Residence |
| Southwest RM-2 Multiple Residence | South RM-2 Beverly Park | Southeast RM-4 Multiple Residence |

Compatibility with Surrounding Land Uses:

Properties to the north are zoned RS-6 and developed with single residences. Existing multiple residences are located west and east of the project site. Bevely Park is located south of the project site and zoned RM-2 and RS-6.

Overall, the proposed development is compatible with existing development in the surrounding area.

PAD Overlay Modification – MZO Article 3, Chapter 22:

Per Section 11-22 of the MZO, the purpose of the PAD overlay is to allow innovative design and flexibility that creates high-quality development for the site where it can be demonstrated that the proposed development provides equivalent or superior standards. Table 1 below shows the MZO required standards, the modified standards approved under the existing PAD, and the applicant’s proposed PAD standards.

Table 1: Development Standards

| Development Standards | MZO Required | Proposed PAD | Staff Recommendation |
| --- | --- | --- | --- |
| Lot Coverage – MZO Table 11-5-5 | 70% | 78% | As Proposed |
| Minimum Yards – MZO Table 11-5-5 - Interior Side and Rear: 3 or more units on lot | 15 feet per story (30 feet total for a two-story building and 45 feet total for a three-story building) | 20 feet minimum (north property line) 8 feet minimum (east property line only) 5 feet (south property line only) | As Proposed |
| Attached Garages – MZO Section 11-5-5(B)(4)(f)(iii) | The maximum number of garage doors adjacent to one another shall be limited to three (3), unless there is a break in the building façade between garage doors. | The maximum number of garage doors adjacent to one another shall be limited to five (5), unless there is an architectural break in the building façade between garage doors. | As Proposed |
| Required Landscape Yard Width – MZO Section 11-33-3(B)(1)(a)(i) & Section 11-33-3(B)(2)(a)(ii) - Non-single residences uses adjacent to single residence uses or districts: sites less than five acres (South property line) (North property line) - Non-single residence uses adjacent to other non-single residence uses or districts (East property line) (South Property line) | 20 feet 20 feet 15 feet 15 feet | 5 feet 15 feet 8 feet 5 feet | As Proposed |
| Setback of Cross Drive Aisles – MZO Section 11-32-4(A) | 50 feet | 26 feet | As Proposed |
| Landscape Island Width – MZO Section 11-33-4(B)(2) | 8 feet wide | 1 foot wide | As Proposed |
| Landscape Island Plant Material – MZO Section 11-33-4(D)(1)(a) | One shade tree and three shrubs | No landscaping | As Proposed |

Lot Coverage:

Per Table 11-5-5 of the MZO, a maximum of 70% of the lot is allowed to be covered by impervious areas. The applicant is proposing to increase the lot coverage to 78% to allow for the proposed product fit on site at the proposed density.

Minimum Yards:

Per Table 11-5-5 of the MZO, a minimum 15-foot interior side and rear yard is required for each story of a multiple residence building. The required setbacks along the north and east property line is 30 feet. The applicant is proposing a 20-foot setback along the northern property line and an eight-foot setback along the eastern property line. The required setback along the south property line is 45 feet. The applicant is requesting a five-foot setback.

Attached Garages:

Per Section 11-5-5(B)(4)(f)(iii) of the MZO, a maximum of three garage doors are allowed on a singular building frontage without a major architectural break. The applicant is requesting five garage doors in a row before an architectural break.

Required Landscape Yard Width:

Per Section 11-33-3(B)(1)(a)(1) of the MZO, a minimum required landscape yard of 20 feet is required for non-single residence uses or districts less than five acres adjacent to single residence uses or districts. The properties north of the Proposed Project are zoned RS-6 and are developed as single residences, while the property to the south of the property is a City of Mesa park zoned RS-6 and RM-2. The applicant is proposing a reduction to the required landscape yard to 15 feet along the north property line and 5 feet for the south property line adjacent to the RS-6 zoned portions of the park.

Additionally, per Section 11-33-3(B)(2)(a)(ii) of the MZO, a 15-foot required landscape yard is required between non-single residence uses and other non-single residence uses, which would be applicable to the east property line and the portions of the south property line adjacent to the RM-2 zoned portions of the park. The applicant is requesting a reduction in the required landscape yard to eight feet along the east property and five feet adjacent to the south property line.

Setback of Cross Drive Aisles:

Per Section 11-32-4(A) of the MZO, parking spaces along main drive aisles connecting directly to a street and drive aisles that cross such main drive aisles shall be set back at least 50 feet from the property line abutting the street. The applicant is requesting to reduce the proposed setback from 50 feet to 26 feet to allow for garage parking for the units closest to Beverly.

Landscape Island Width:

Per Section 11-33-4(B)(2) of the MZO, a minimum width of eight feet is required for all landscape islands. The applicant is requesting to reduce the minimum width of landscape islands to one foot in order to increase the open space areas elsewhere on site. The proposed deviations would affect the landscape islands in the parking areas adjacent to the proposed amenity area as well as adjacent to the solid waste enclosure.

Landscape Island Plant Material:

Per Section 11-33-4(D)(1)(a) of the MZO, one shade tree and three shrubs are required to be planted in a parking lot landscape island. The applicant is requesting to eliminate the landscaping requirement as a one-foot-wide landscape island would not accommodate landscaping.

Justification:

The applicant’s justification for the requested deviations includes an increased amount of open space and the unique building architecture. Per table 11-5-5 of the MZO, a minimum of 150 square feet of open space per unit is required in the RM-4 zoning district. Based on the 68 units proposed, this would equate a total of 10,200 square feet of required open space. Per the plans submitted, the applicant is providing a total of 19,135 square feet of open space within the development. In addition to private open space provided with each unit, the plans submitted also show common open space amenities including a pool, barbeque area and dog park.

Site Plan and General Site Development Standards:

The proposed site plan shows the development of six multiple residence buildings containing a mixture of two- and three-bedroom units for a total of 68 units.  All units contain private open space (i.e., balcony or ground floor patio) that complies with the minimum size and dimensional standards set forth in MZO. A large central common open space area is shown on the southern portion of the site between the residential buildings and Beverly Park which contains a pool, dog park, and BBQ ramada.

Access to the site is provided from North Beverly along the west side of the property and emergency access is provided from Beverly along the northwest side of the property. Per Table 11-32-3.A of the MZO, 2.1 parking spaces are required per unit for a total of 143 required parking spaces.  A total of 157 parking spaces are provided throughout the site.

Overall, the proposed site plan complies with the review criteria for Site Plan Review in Section 11-69-5 of the MZO.

Design Review:

The Design Review Board reviewed the proposed building elevations and landscape plan on January 14, 2025. The Design Review Board had several concerns about the proposed building elevations and the impact of the overall site plan on the neighboring property owners. The Board had the following comment or recommendations:

- Removal of the proposed parallel parking spots along the northern property line to increase the proposed landscape yards to improve the screening for the neighboring properties.

- Concerns about the width of the proposed north/south pedestrian corridors that bisected the buildings and how the location of SES panels within those corridors would impact the overall walkability of the site.

- Concerns for the lack of foundation base width to support landscaping.

- Concerns about the location of the trash enclosure next to the amenity area.

- Concerns about the proposed elevations along the garage sides of the units. The Board requested that the applicant continue the architectural element adjacent to Beverly throughout the elevations to provide for four-sided architecture.

The applicant revised the site plan to remove the parallel parking spaces along the northern property line and increase the landscape yard. The north/south pedestrian corridors between the buildings were also removed which allowed for increased foundation base for landscaping and the shifting of building to allow for the relocation of the trash enclosures.

The applicant is working with staff to address the comments related to the elevations.

#### Neighborhood Participation Plan and Public Comments:

The applicant completed a Citizen Participation Process, which included inviting property owners within 1,000 feet of the site, as well as HOAs within ½ mile, and registered neighborhood within one mile.

A virtual neighborhood meeting on September 5, 2024, to allow interested parties the opportunity to ask questions and provide opinions on the Proposed Project. Per the Citizen Participation Report, there were 12 attendees of the neighborhood meeting. Neighboring property owners had concerns about the proposed density and increase to traffic in the area as well as overall possible congestion. The citizens also had concerns about the Proposed Project had on the existing foliage on site that created greater screening from the park, whose safety facilities are a concern of the neighborhood due to activities that occur in the park outside the park hours.

The applicant provided a traffic study which shows the overall impact of the Proposed Project on the traffic network as well as increasing landscaping areas to provide for more foliage to screen the new units while also creating more visibility for the park to provide for more eyes on the street to increase the natural surveillance.

As of the writing of this report, staff has only received general inquires about this proposal. Two neighboring property owners had questions whether the development was going to be market rate or affordable housing. Staff will provide the Board with any new information during the scheduled Study Session on January 22, 2024.

School Impact Analysis:

City of Mesa staff provided project information to Mesa Public Schools and requested information on the anticipated impacts of the project and capacity of local schools.  As of writing this report, staff has not received any comments Mesa Public Schools.

Staff Recommendation:

Staff finds the subject request is consistent with the Mesa 2040 General Plan, the review criteria for Site Plan Review outlined in Section 11-69-5 of the MZO, and the review criteria for a Planned Area Development Overlay outlined in Section 11-22-3(B) of the MZO.

Staff recommends approval with the following Conditions of Approval:

- Compliance with the final site plan submitted.

- Compliance with all requirements of Design Review Case No. DRB24-00639.

- Prior to building permit submittal, review and approval by City of Mesa staff of the proposed major architectural feature provided between every five garages.

- Compliance with all City development codes and regulations, except the modifications to the development standards as approved with this PAD and shown in the following table:

| Development Standards | Approved |
| --- | --- |
| Lot Coverage – MZO Table 11-5-5 | 78% |
| Minimum Yards – MZO Table 11-5-5 - Interior Side and Rear: 3 or more units on lot | 20 feet minimum (north property line) 8 feet minimum (east property line only) 5 feet (south property line only) |
| Attached Garages – MZO Section 11-5-5(B)(4)(f)(iii) | The maximum number of garage doors adjacent to one another shall be limited to five (5), unless there is an architectural break in the building façade between garage doors. |
| Required Landscape Yard Width – MZO Section 11-33-3(B)(1)(a)(1) & Section 11-33-3(B)(2)(a)(ii) - Non-single residences uses adjacent to single residence uses or districts: sites less than five acres (South property line) (North property line) - Non-single residence uses adjacent to other non-single residence uses or districts (East property line) (South Property line) | 5 feet 15 feet 8 feet 5 feet |
| Setback of Cross Drive Aisles – MZO Section 11-32-4(A) | 26 feet |
| Landscape Island Width – MZO Section 11-33-4(B)(2) | 1 foot wide |
| Landscape Island Plant Material – MZO Section 11-33-4(D)(1)(a) | No landscaping |

Exhibits:

Exhibit 1 – Staff Report

Exhibit 2 – Vicinity Map

Exhibit 3 – Project Narrative

Exhibit 4 – Site Plan

Exhibit 5 – Landscape Plan

Exhibit 6 – Preliminary Grading and Drainage Plan

Exhibit 7 – Elevations

Exhibit 8 – Citizen Participation Plan

Exhibit 9 – Citizen Participation Report

Exhibit 10 – PowerPoint Presentation